DESIGN OF NANO ZINC OXIDE–JANIA RUBENS ECO–FRIENDLY MATRIX FOR REMOVAL OF CADMIUM (II): CHARACTERIZATION AND DESORPTION STUDIES

Kishore Kumar Kadimpati a,b*, Sujatha Sanneboina b, Abhishek Duttac,d, Krishna Prasad Mondithokae, Laura Bulgariuf

a Osmania University, Mallareddy College of Pharmacy, Department of Pharmaceutics, Dhullapally: Secunderabad-500100, Telangana, India

b JNTU Anantapuram, Narayana Pharmacy College, Department of Pharmaceutics, Nellore-524003, Andhra Pradesh, India
c KU Leuven, Faculteit Industriële  Ingenieurswetenschappen, Campus Groep T Leuven, Andreas Vesaliusstraat 13, 3000 Leuven, Belgium

d KU Leuven, Departement Metaalkunde, Kasteelpark Arenberg 44 Bus 2450, 3001 Heverlee, Belgium

e JNTU Kakinada, G.M.R. Institute of Technology, Department of Chemical Engineering, Rajam-532127, Andhra Pradesh, India

fTechnical University Gheorghe Asachi of Iasi, Faculty of Chemical Engineering and Environmental Protection, Department of Environmental Engineering and Management, Iasi-700050, Romania

E-mail: drkadimpatikks@gmail.com (corresponding author)

Abstract. In this study ZnO–J. rubens–PVA–A matrix using nano zinc oxide particles were synthesized, which is fixed onto marine red algae biomass, Jania rubens, embedded in Polyvinyl alcohol–alginate–matrix (PVA–A) and used for removal of cadmium (Cd(II)) ions. This hybrid matrix was characterized using DLS, FTIR, SEM and EDAX. The equilibrium time was calculated to be 2 h and the favorable pH was found to be 5. The adsorption capacity (qe) of matrix increased from 10.47 to 66.40 mg g–1 with initial Cd(II) amount increasing from 25.25 to 152.96 mg L–1 at a sorbent dose of 2 g L–1 at 303 K and pH 5. The isotherm data fitted well with both the Langmuir and the Freundlich models, and RL values varied between 0.326-0.185 while the maximum Cd(II) biosorption (qm) was calculated to be 111 mg g–1. The matrix could be efficiently re-used for three cycles with no significant loss in biosorption capacity.

Keywords: nano-ZnO, Jania rubens, thermodynamics, pseudo second order kinetics, desorption.