Nanocomposites and nanomaterials ("Nanotechnology and nanomaterials")

Electrical Properties of Polymethyl methacrylate (PMMA) and PSy-g-PMMA Graft Copolymers

Alli A.1, Alli S.2, Demir A.3

1 Department of Chemistry, Düzce University Düzce-81620, Turkey.

2 Department of Polymer Engineering, Düzce University Düzce-81620, Turkey.  E-mail: semaalli@duzce.edu.tr

3 Department of Physics, Düzce University Düzce-81620, Turkey.

Organic field-effect transistors (OFETs) have attracted attention due to their low budget requirements when they fabricate and their mechanical flexibility properties [1]. Unlike silicon transistors, they can be fabricated by simple techniques such as spin coating, casting and inkjet printing with a reasonable low price [2].

In this study, soya oil was exposed to air under daylight at room temperature in order to obtain polymeric soya oil peroxide (PSy-peroxide). PSy-g-PMA graft copolymers which is a new partially biodegradable material were obtained by free radical polymerization of poly(methyl acrylate) (PMA)  using PSy-peroxide as macroinitiator.

Two non-ionic gel-OFETs with different gate insulators which are formed by mixing dissolved PMA and PSy-g-PMA polymers with PC separately and stirring all solutions until becoming a gel was fabricated and compared. However, OFET with PSy-g-PMA as well as showed the best output characteristic which is offset-free compared to the other transistors it has the highest on-current that can be extracted from its transfer characteristic.

Acknowledgments: The Duzce University Research Fund (Grant Number: 2015.05.03.381, 2018.05.03.674, 2018.05.03.681).

- I. Kymissis Organic Field Effect Transistors: Theory, Fabrication and Characterization // Springer Science & Business Media, 2008.

- Y. Sun, Y. Liu, and D. Zhu, Advances in organic field-effect transistors // J Mol Biol.-2005.- 15, N 1.-P. 53-65.