Nanocomposites and nanomaterials ("Nanotechnology and nanomaterials")

Investigation of Polymeric Linoleic Acid Graft Copolymer as the organic field effect transistor (OFET) applications

Allı A.1, Allı S.2, Gündüz M.1, Demir A.3

1 Department of Chemistry, Düzce University Düzce-81620, Turkey.

2 Department of Polymer Engineering, Düzce University Düzce-81620, Turkey.  E-mail: semaalli@duzce.edu.tr

3 Department of Physics, Düzce University Düzce-81620, Turkey.

Block/graft copolymers have been successfully synthesized by a one-pot polymerization technique, which indeed has several advantages over other widely employed methods [1-2]. Organic field effect transistors (OFETs) have been a significant research topic over decades. It seems that their importance will be increased in the future, because of they have a low thermal budget when fabricating and great mechanical flexibility properties [3].

In the current work, a novel graft copolymers having structures of poly(linoleic acid)-g-poly(methyl methacrylate)-g-poly(D,L-lactide) (PLina-g-PMMA-g-PLA) were synthesized from polymeric linolenic acid, possessing peroxide groups on the main chain by the combination of free radical polymerization of methyl methacrylate and ring-opening polymerization of D,L-lactide in one-pot. Graft copolymer was used as a insulator on a prepatterned the organic field effect transistor (OFET) substrate. A novel polymer insulator was used in ITO/Polymer/PEDOT:PSS structure to obtain as a OFET devise. It exhibited good device performance such as low VTh, comparable mobility, and  Ion/off.

Acknowledgments: The Duzce University Research Fund (Grant Number: 2015.05.03.381, 2018.05.03.674, 2018.05.03.681).

- Allı A., Allı S., Becer C. R., Hazer H., One-pot synthesis of poly(linoleic acid)-g-poly(styrene)-g-poly(ε-caprolactone) graft copolymers // Journal of The American Oil Chemists Society.-2014.-91,N 5.-P. 849-858.

- Öztürk T., Göktaş M., Hazer B., One-Step Synthesis of Triarm Block Copolymers via Simultaneous Reversible-Addition Fragmentation Chain Transfer and Ring-Opening Polymerization // J. Macromol. Sci. Part A. Pure and Appl. Chem.- 2010.-117, N 3.-P. 1638-1645.

3.  I. Kymissis Organic Field Effect Transistors: Theory, Fabrication and Characterization // Springer Science & Business Media, 2008.